MODULE 41
CUDA Basics

GPU kernels with Numba + CuPy + a peek at Triton
Part of the Data Science Roadmap · Module 41

🎯 Why this module matters: every fast LLM kernel — FlashAttention, Mamba, Unsloth's training kernels — bottoms out in CUDA. You don't need to write production CUDA C++ to be effective, but you DO need the execution-model intuition: threads, warps, blocks, grids, and the memory hierarchy. This module gives you that intuition by writing real kernels in Python via Numba.

Prerequisites: M16 (PyTorch basics). GPU runtime required.

What you'll cover

1. Why GPUs are fast — parallelism, not clock

2. Execution model: thread → warp → block → grid → SM

3. Memory hierarchy: registers / shared / global

4. First kernel — vector add (@cuda.jit)

5. 2-D indexing — image kernel

6. Shared memory — tiled matmul

7. CuPy — NumPy on GPU

8. Profiling — events + nvidia-smi

9. Bridging to PyTorch + Triton

10. When to write CUDA vs let the framework handle it

# 1. Why GPUs are fast

|  | CPU | GPU |
| --- | --- | --- |
| Cores | ~16 | thousands |
| Clock | ~5 GHz | ~1.5 GHz |
| Memory bandwidth | ~50 GB/s | ~300–3000 GB/s |
| Best at | branchy serial code | dense numerics, same op on millions of elements |

🔑 Mental model: the CPU is a sports car (one driver, fast lap). The GPU is a freight train (thousands of carriages, slow per carriage but moves enormous volume). ML batches are freight, not laps.

# 2. Execution model

| Term | What it is |
| --- | --- |
| Thread | one logical execution stream; runs your kernel |
| Warp | 32 threads in lockstep (SIMD); the real scheduling unit |
| Block | ≤1024 threads; share SHARED MEMORY and can synchronise |
| Grid | the whole launch — many blocks |
| SM | Streaming Multiprocessor; hardware unit running many warps concurrently |

Launch syntax: kernel[blocks_per_grid, threads_per_block](args). Each thread sees cuda.threadIdx, cuda.blockIdx, cuda.blockDim, cuda.gridDim — that's how it figures out WHICH element of the input to process.

# 3. Memory hierarchy

| Level | Size | Latency | Scope |
| --- | --- | --- | --- |
| Registers | ~64 KB / SM | 1 cycle | per thread |
| Shared memory | ~48–96 KB / block | ~5 cycles | per block |
| L1 / L2 | MB | tens of cycles | per SM / GPU |
| Global (HBM) | GB | 100s cycles | whole GPU |
| Host (CPU RAM) | GB | µs over PCIe | CPU |

🔑 The single biggest CUDA performance lesson: load data from GLOBAL into SHARED memory ONCE, then reuse it many times inside the block. That's the whole story behind FlashAttention.

# 4. First kernel — vector add

| ●1 | from numba import cuda |
| --- | --- |
| ●2 |  |
| ●3 | @cuda.jit |
| ●4 | def vec_add(a, b, c): |
| ●5 | i = cuda.grid(1) |
| ●6 | if i < a.size: |
| ●7 | c[i] = a[i] + b[i] |
| ●8 |  |
| ●9 | threads_per_block = 256 |
| ●10 | blocks_per_grid = (N + threads_per_block - 1) // threads_per_block |
| ●11 | vec_add[blocks_per_grid, threads_per_block](a, b, c) |

| ●1 | Numba's CUDA module — JIT-compiles Python to PTX |
| --- | --- |
| ●2 | Decorator: this Python function is a CUDA kernel |
| ●3 | Kernel signature — kernels return nothing; they write into output arrays |
| ●4 | Global thread index along dim 0 — same as blockIdx.x*blockDim.x + threadIdx.x |
| ●5 | Bounds check — last block usually has extra threads |
| ●6 | The actual work: one element per thread |
| ●7 | 256 threads per block is a sensible default |
| ●8 | Ceiling division — ensures we cover all N elements |
| ●9 | Launch: block-grid first, then thread-block; args in parens like a normal call |

# 5. 2-D indexing

| ●1 | @cuda.jit |
| --- | --- |
| ●2 | def grayscale(rgb, gray): |
| ●3 | x, y = cuda.grid(2) |
| ●4 | if x < rgb.shape[0] and y < rgb.shape[1]: |
| ●5 | r, g, b = rgb[x, y, 0], rgb[x, y, 1], rgb[x, y, 2] |
| ●6 | gray[x, y] = 0.299*r + 0.587*g + 0.114*b |

| ●1 | Same decorator |
| --- | --- |
| ●2 | Kernel: input RGB, output gray |
| ●3 | cuda.grid(2) returns a 2-tuple — perfect for image kernels |
| ●4 | 2-D bounds check |
| ●5 | Read the three channels |
| ●6 | ITU-R 601 luma coefficients |

# 6. Shared memory — tiled matmul

Naïve matmul reads each element of A and B from global memory N times. The TILED version loads a tile into shared memory ONCE and every thread in the block reuses it. This is the single most important CUDA pattern — it's how FlashAttention works at the macro level.

| ●1 | sA = cuda.shared.array((TPB, TPB), float32) |
| --- | --- |
| ●2 | sB = cuda.shared.array((TPB, TPB), float32) |
| ●3 | ... |
| ●4 | for tile in range((A.shape[1] + TPB - 1) // TPB): |
| ●5 | sA[tx, ty] = A[x, tile*TPB + ty] |
| ●6 | sB[tx, ty] = B[tile*TPB + tx, y] |
| ●7 | cuda.syncthreads() |
| ●8 | for k in range(TPB): |
| ●9 | acc += sA[tx, k] * sB[k, ty] |
| ●10 | cuda.syncthreads() |

| ●1 | Allocate a tile of A in shared memory (per-block scratchpad) |
| --- | --- |
| ●2 | Same for B |
| ●3 | (elided init — see notebook) |
| ●4 | Iterate over tiles along the K dimension |
| ●5 | Cooperative load: each thread brings in one element of A's tile |
| ●6 | Same for B's tile |
| ●7 | Barrier — wait for all threads in the block to finish loading |
| ●8 | Accumulate the dot-product within the tile |
| ●9 | Multiply-add |
| ●10 | Barrier before overwriting tiles in the next iteration |

# 7. CuPy — NumPy on GPU

90% of the time you don't need a custom kernel — you just want NumPy on the GPU. CuPy is a drop-in replacement: same API, runs on CUDA, ~50-200× faster on big arrays.

| ●1 | import cupy as cp |
| --- | --- |
| ●2 | x_gpu = cp.asarray(x_cpu) |
| ●3 | _ = x_gpu @ x_gpu |
| ●4 | cp.cuda.Stream.null.synchronize() |

| ●1 | Import — same shape as numpy |
| --- | --- |
| ●2 | Move data to GPU (this is a copy across PCIe — do it once, not per op) |
| ●3 | Matmul on GPU — runs cuBLAS underneath |
| ●4 | GPU work is async; sync to time accurately |

# 8. Profiling

| ●1 | start = cuda.event(); end = cuda.event() |
| --- | --- |
| ●2 | start.record() |
| ●3 | vec_add[blocks_per_grid, threads_per_block](a, b, c) |
| ●4 | end.record(); end.synchronize() |
| ●5 | print(f"vec_add took {cuda.event_elapsed_time(start, end):.3f} ms") |

| ●1 | CUDA events bracket a kernel for accurate timing |
| --- | --- |
| ●2 | Mark the start |
| ●3 | Run the kernel under measurement |
| ●4 | Mark the end and wait for completion |
| ●5 | Read elapsed wall time in milliseconds |

Occupancy = (active warps per SM) / (max warps per SM). Higher occupancy hides memory latency. Real tools: NSight Compute (ncu), CUPTI, numba.cuda.detect().

# 9. Bridge to PyTorch + Triton

PyTorch already exposes thousands of tuned CUDA kernels (matmul, scaled_dot_product_attention, conv2d). You only write a custom kernel when (1) the op doesn't exist yet, (2) you're fusing ops to avoid global-memory round-trips, or (3) you're squeezing the last few percent of a hot path.

TRITON (OpenAI) is the modern answer to 'I want CUDA performance from Python'. Looks like NumPy + decorators, compiles to PTX. Most new ML kernels (FlashAttention-3, Mamba, Unsloth) ship as Triton, not raw CUDA.

# 10. When to write CUDA

| Situation | What to do |
| --- | --- |
| Standard ML ops (matmul, conv, attn) | Use PyTorch/TF — already optimal |
| Custom op, performance matters | Write it in Triton — 90% of CUDA, 10× less code |
| Critical hot path, every µs counts | Drop to CUDA C++ + cuBLAS / cuDNN |
| NumPy-on-GPU | CuPy — done in one line |
| Multi-GPU training | NCCL via torch.distributed, not custom |
| Edge device (mobile, embedded) | Don't write CUDA; export to TFLite / ONNX / TensorRT |

# Recap

• GPU = thousands of cores running the same kernel on different data

• kernel[blocks, threads](args) — total threads = blocks × threads

• Memory hierarchy is the WHOLE performance story — load to shared once, reuse

• CuPy = NumPy-on-GPU; Numba = CUDA from Python; Triton = modern fast ML kernels

• Don't write CUDA unless PyTorch / Triton can't already