MODULE 77
GPU Networking

NVLink · NVSwitch · InfiniBand · RoCE · GPUDirect RDMA
Phase 10 · Master-Plan Gaps · Module 77

🎯 Why this module matters: M66 said 'all-reduce gradients across GPUs.' This module is HOW. Below your torch.distributed call sits a stack: NCCL → CUDA IPC → NVLink in-node → RDMA over InfiniBand or RoCE between nodes. Get any layer wrong and your 256-GPU training run becomes a 100-GPU run with 156 GPUs idle. This is the network half of distributed training.

Prerequisites: M66 (distributed training), M46 (Kubernetes / GPU scheduling), basic networking.

What you'll cover

1. The hierarchy — why all-reduce is a multi-layer problem

2. NCCL collectives

3. NVLink + NVSwitch — intra-node

4. PCIe — the slow fallback

5. InfiniBand — the gold standard

6. RoCE v2 — RDMA over Ethernet

7. GPUDirect RDMA — wire-speed GPU↔GPU

8. Rail-optimised topology

9. Tuning NCCL — env vars + nccl-tests + topo -m

10. The 2025-26 fabric landscape

# 1. The hierarchy

● 1  Python                  loss.backward(); optimizer.step()

● 2  PyTorch / JAX        torch.distributed → all_reduce

● 3  NCCL                 ring / tree / double-binary-tree

● 4  INTRA-NODE          NVLink ← NVSwitch fabric → GPU 0..7

● 5  INTER-NODE          GPUDirect RDMA → IB HCA → IB switch (fat-tree)

● 6                        (or) GPUDirect RDMA → ConnectX RoCE NIC → AI-class Ethernet

| ● | What it does |
| --- | --- |
| ●1 | Your training code at the top |
| ●2 | Framework dispatches to torch.distributed |
| ●3 | NCCL picks the algorithm + fabric path |
| ●4 | Inside one box: NVLink at hundreds of GB/s |
| ●5 | Between boxes: RDMA over IB or RoCE |
| ●6 | Two equivalent inter-node options |

🔑 The fabric is NOT abstracted away. When training slows, fix it here — not in your model code.

# 2. NCCL collectives

| Collective | Where it shows up |
| --- | --- |
| AllReduce | DDP gradient sync (M66 §3) |
| AllGather | FSDP unsharding (M66 §4) |
| ReduceScatter | FSDP gradient sharding |
| Broadcast | initial weight spread |
| AlltoAll | MoE routing, sequence-parallel |
| Send/Recv | pipeline-parallel boundary passes |

🔑 Ring all-reduce on N GPUs moves 2(N-1)/N of the buffer per link — INDEPENDENT of N. That's why it scales. NCCL_DEBUG=INFO prints the chosen algorithm + topology at startup.

# 3. NVLink + NVSwitch

| Gen | Per-link | Per-GPU agg. | Used by |
| --- | --- | --- | --- |
| NVLink 3 | 50 GB/s | 600 GB/s | A100 |
| NVLink 4 | 100 GB/s | 900 GB/s | H100 / H200 |
| NVLink 5 | 200 GB/s | 1.8 TB/s | B100 / B200 / GB200 |

🔑 NVSwitch turns the 8-GPU node into a fully-connected mesh — every GPU has full per-link bandwidth to every other GPU simultaneously. That's why the 8× H100 box is the unit of frontier-LLM purchases. Check yours with `nvidia-smi topo -m` — look for NV-something, PIX is bad, SYS is a disaster.

# 4. PCIe — slow fallback

| PCIe gen | Per-lane | x16 GPU |
| --- | --- | --- |
| Gen 3 | 1 GB/s | 16 GB/s |
| Gen 4 | 2 GB/s | 32 GB/s |
| Gen 5 | 4 GB/s | 64 GB/s |
| Gen 6 | 8 GB/s | 128 GB/s |

🚫 PCIe Gen 5 x16 is ~14× slower than NVLink 5. If `nvidia-smi topo` shows PCIe between GPUs you wanted to talk fast → fix it before training. On dual-socket boxes, ALSO check NUMA with numactl/lstopo — a GPU on socket 0 hitting a NIC on socket 1 crosses the inter-socket link.

# 5. InfiniBand

| Gen | Per-port | Used by |
| --- | --- | --- |
| HDR | 200 Gb/s | A100-era |
| NDR | 400 Gb/s | H100 reference |
| XDR | 800 Gb/s | B100 / B200 / GB200 |

Three properties that make IB right for ML training:

• RDMA — NIC writes directly into remote GPU memory; no CPU involvement

• Lossless — credit-based flow control; frozen receivers back-pressure, never drop

• Fat-tree topology — non-blocking; every leaf has full bandwidth to every other leaf

🔑 HCA = the IB NIC (ConnectX-7 on H100). opensm = subnet manager. PKey ≈ VLAN, GUID ≈ MAC.

# 6. RoCE v2

RDMA over Ethernet — same RDMA semantics on standard 400/800 GbE. Requirements:

• PFC (Priority Flow Control) — makes Ethernet lossless for the RDMA class

• ECN + DCQCN — congestion control

• Tuned switch buffers — absorb micro-bursts

• Single L3 domain or tight VXLAN — minimise hops

| Criterion | InfiniBand | RoCE v2 |
| --- | --- | --- |
| Bandwidth | up to XDR 800 Gb/s | up to 800 Gb/s Eth |
| Latency | best-in-class (sub-µs) | ~1.5-3× IB |
| Cost | $$$ | $$ |
| Ops | parallel fabric to manage | shares IT Ethernet skill |
| Hyperscaler use | OpenAI / Anthropic / MS / Meta RSC1 | Meta RSC2, Azure ND, Oracle OCI |
| Multi-tenant | mature (PKeys) | newer; careful PFC scoping |
| Switches | NVIDIA Quantum-2 | Spectrum-X / Tomahawk 5 / Cisco Nexus AI |

🔑 2026 default: new small/mid clusters (<1000 GPUs) → RoCE v2 for cost. Large frontier clusters → InfiniBand NDR/XDR for absolute performance. Meta's RSC2 (24K H100s on Ethernet) proves RoCE can scale.

# 7. GPUDirect RDMA

Without GPUDirect: GPU → pinned host RAM → NIC → wire → NIC → pinned host RAM → GPU. Four copies, two CPUs, wasted bus. With GPUDirect: NIC reads directly from GPU memory. Zero CPU involvement, wire-speed.

● 1  $ modinfo nvidia-peermem

● 2  $ ibv_devinfo

● 3  $ ucx_info -d | grep -i cuda

| ● | What it does |
| --- | --- |
| ●1 | Verify the kernel module that lets the NIC see GPU pages |
| ●2 | List IB / RoCE devices visible to the userspace driver |
| ●3 | Confirm UCX (NCCL's transport layer) reports CUDA support |

🚨 If GPUDirect isn't working, NCCL silently falls back to host-staging → ~3× slowdown — easy to miss until you profile. Always verify post-install: nvidia-peermem, MOFED / DOCA-OFED, NCCL ≥ 2.16 with mlx5 + CUDA build.

# 8. Rail-optimised topology

The trick that makes 8-GPU servers scale into 1024-GPU pods: GPU i in EVERY node connects to the SAME rail of the fabric. So GPU 0 in node A talks to GPU 0 in node N over the shortest path. AllReduce traffic between matching ranks never crosses the spine more than necessary.

🔑 Reference designs that bake this in: DGX H100 / H200 / GB200 NVL72, HGX H100/H200/B100/B200, SuperPOD. Cloud: AWS p5 / p6, GCP A3 / A3-mega / A3 Ultra, Azure ND H100 v5.

# 9. Tuning NCCL

● 1  export NCCL_DEBUG=INFO

● 2  export NCCL_SOCKET_IFNAME=eth0,eth1

● 3  export NCCL_IB_HCA=mlx5_0,mlx5_1

● 4  export NCCL_IB_GID_INDEX=3

● 5  export NCCL_NET_GDR_LEVEL=PHB

● 6  export NCCL_NVLS_ENABLE=1

● 7  export NCCL_P2P_DISABLE=0

● 8  export NCCL_IB_TIMEOUT=22

● 9  export NCCL_ASYNC_ERROR_HANDLING=1

| ● | What it does |
| --- | --- |
| ●1 | Print topology + algorithm choices at startup — turn on for first run |
| ●2 | Out-of-band (control) network interface — NOT data |
| ●3 | Which IB / RoCE HCAs to use, comma-separated |
| ●4 | RoCE v2 IPv6 GID index; verify with `show_gids` |
| ●5 | Require GPUDirect RDMA up to this distance (PHB, PIX, ...) |
| ●6 | Use NVSwitch multicast (NVLink Sharp) on H100/H200/B200 |
| ●7 | Disable P2P only to debug |
| ●8 | Raise IB timeout on flaky fabrics |
| ●9 | Don't hang forever on partial failures |

● 1  git clone https://github.com/NVIDIA/nccl-tests

● 2  cd nccl-tests && make MPI=1

● 3  mpirun -np 16 -hostfile hostfile \

● 4      ./build/all_reduce_perf -b 8 -e 8G -f 2 -g 1

| ● | What it does |
| --- | --- |
| ●1 | Canonical NCCL benchmark suite |
| ●2 | Build with MPI for multi-node |
| ●3 | Launch across 16 ranks on the listed hosts |
| ●4 | Args: byte range, factor, 1 GPU per rank |

| Step | Check |
| --- | --- |
| nvidia-smi topo -m | every GPU sees NV-something to every other (NV12+) |
| ibstat / mlxconfig | HCAs up, at advertised speed |
| all_reduce_perf intra-node | matches NVSwitch spec (~330+ GB/s busbw on 8x H100) |
| all_reduce_perf inter-node | ~80% of NIC line rate (~40+ GB/s on NDR) |
| nsys profile a training step | NCCL collectives overlap with compute |

🔑 Read the busbw column from nccl-tests. If it's an order of magnitude off the expected number → STOP training, fix networking first. Most 'distributed-training is slow' tickets are network problems, not model problems.

# 10. The 2025-26 landscape

● 1  2020   A100         NVLink 3   600 GB/s/GPU    HDR 200 Gb/s

● 2  2022   H100         NVLink 4   900 GB/s/GPU    NDR 400 Gb/s

● 3  2024   H200         NVLink 4   900 GB/s/GPU    NDR 400 Gb/s

● 4  2025   B100/B200    NVLink 5   1.8 TB/s/GPU    XDR 800 Gb/s

● 5  2025+  GB200 NVL72  NVLink 5   +130 TB/s agg.  XDR 800 Gb/s

| ● | What it does |
| --- | --- |
| ●1 | Year + GPU + intra-node + inter-node generations |
| ●2 | Each row doubles or quadruples some axis |
| ●3 | (continuation) |
| ●4 | (continuation) |
| ●5 | GB200 NVL72 blurs the intra/inter-node boundary |
| ●6 | 72 Blackwells in one NVLink domain via copper + switch tray |

| Cluster size | Suggested fabric |
| --- | --- |
| Sub-1000 GPUs | NVLink intra-node + 100/200 Gb RoCE inter-node |
| 1000-10 000 GPUs | NVLink intra + NDR or XDR IB inter, rail-optimised |
| > 10 000 GPUs | Custom (Meta RSC2 = Ethernet; OpenAI / Anthropic = IB) |

🔬 Next-gen: CXL 2.0/3.0 is the cache-coherent bus for memory pooling. Spectrum-X with Bluefield-3 adds CPU-offload + telemetry on the Ethernet side. Quantum-2/3 is NVIDIA's IB switch line.

# Recap

• All-reduce is a stack: NCCL → CUDA IPC → NVLink in-node → RDMA over IB/RoCE between nodes

• NCCL picks the algorithm; you provide topology, env vars, and GPUDirect

• NVLink + NVSwitch = bandwidth-rich mesh inside one box

• InfiniBand NDR/XDR = gold standard inter-node; RoCE v2 on Spectrum-X = cost-effective Ethernet

• GPUDirect RDMA is MANDATORY — verify post-install or lose 3× silently

• Rail-optimised fat-trees scale to thousands of GPUs

• Tune with NCCL_DEBUG=INFO + nccl-tests + nvidia-smi topo -m. Fix networking before model code